Media Contact:

Pablo Gonzalez-Huerta

PR Communications Assistant Manager

Tel: +49 69 850 928 342

E-mail: pghuerta@kia-europe.com

Press kit

For immediate release

The Kia Picanto: a distinctive new design with ‘class-above’ technologies

- Refreshed design for all models, including GT-Line and X-Line variants

- A vibrant character: new interior colour customisation options available

- Efficient new ‘Smartstream’ gasoline powertrain line-up and a new Automated Manual Transmission option for drivers needing an auto

- New ‘Phase II’ UVO Connect telematics and advanced infotainment system

- High-tech safety features include Lane Following Assist and Forward Collision-Avoidance Assist

- Upgraded Picanto on-sale in Europe from Q3 2020

Kia’s smallest model, the Kia Picanto, features a range of substantial mid-life upgrades, keeping it one step ahead of its competitors. The upgraded model boasts a vibrant design and fun-driving attitude, combined with ‘class-above’ connectivity and safety technologies, and lower emissions from newly reengineered powertrains.

The upgraded Picanto brings with it a suite of advanced safety technology from Kia’s larger cars, making it among the safest cars in its class. It also offers Kia’s new ‘Phase II’ UVO Connect telematics technologies, enhancing vehicle connectivity and control to take the stress out of journeys. Enhancements to the Picanto’s gasoline powertrain line-up include Kia’s optional new Automated Manual Transmission (AMT) for a smoother drive.

The Picanto is one of Kia’s best-selling models in several regions worldwide, and a particular favourite among European A-segment drivers. The upgraded Picanto offers Kia’s industry-leading 7-Year, 150,000-kilometre warranty as standard.

Exterior design

Refreshed design with a distinct new look for GT-Line and X-Line models

The Picanto line-up has received a comprehensive enhancement to exterior design, with significant updates made to sporty GT-Line and SUV-inspired X-Line models in particular. The result is a refreshed, more distinctive appearance.

The ‘face’ of both the GT-Line and X-Line Picanto models has been revised to give each car a more purposeful design. This starts with the ‘tiger-nose’ grille, which features a new textured surround and open-link appearance. The chrome-effect surround forms a sweeping shape which extends into the lower part of the redesigned headlamps, and gives the car a wider, more stable appearance. Each side of the ‘tiger-nose’ features a red highlight on GT-Line models, or more technical black trim X-Line models which resemble air intakes.

Both versions of the car now feature redesigned headlights and four-bulb LED daytime running lights. At the rear, the car also features a new LED rear combination lamp design and a slimmer fog light design, creating a more recognisable rear light signature at the same time.

Both GT-Line and X-Line models feature refreshed front and rear bumper designs with sharper creases in the bodywork for a more contemporary appearance. The changes create a more rugged, robust-looking design for X-Line models and, for GT-Line models, an appearance which visually links the car to higher-performance GT models in Kia’s range. A larger, wider air intake in the lower bumper is accented by the GT-Line’s body-coloured bumper with gloss black highlights, and jewel-like fog lamps. X-Line models echo the appearance of SUV protective skid plates, with metal-effect trim bearing the X-Line logo, and a gloss black surround which extends from the front grille into the wheel arch surrounds at the side of the car.

The upgraded Picanto offers buyers greater potential for customisation, with a choice of 10 vibrant paint finishes designed to make the car stand out.

Interior design

Improved infotainment screens and new customisation options

Inside, the Picanto features a series of visual and technological upgrades, centred around a larger 8.0-inch ‘floating’ widescreen audio visual navigation (AVN) system (see ‘Infotainment’ for more details), and a high-resolution 4.2-inch digital display in the driver’s instrument cluster.

Depending on trim level and market, the Picanto’s cabin is offered with black cloth or synthetic leather seats, with the upgraded model finished with a range of new upholstery textures for a more contemporary, higher quality look and feel. The cabin is finished with black, silver or gloss black highlights in the door, on the dashboard and around the gear selector.

The upgraded Picanto is also available with a range of trendy interior colour packs to enhance customers’ ability to customise the cabin of their car, depending on vehicle specification and market.

The Lime Pack features a black single-tone interior with lime contrast stitching and white seat bolsters, silver metal painted transmission and air vent surrounds, and silver-and-lime accents in the doors. Models equipped with the Orange Pack feature a black single-tone interior with orange stitching and cabin highlights, and black-and-orange door accents. Red Pack models are finished in black upholstery with red seat bolsters, as well as gloss black highlights and red-and-black door accents. Finally, the Green Pack offers green highlights throughout the cabin and silver-and-green door accents, as well as stylish denim-effect seat upholstery.

Powertrains

New ‘Smartstream’ engine innovations increase efficiency

The upgraded Picanto is available in Europe with three new ‘Smartstream’ gasoline engines, each maximising efficiency and lowering CO2 emissions compared to their predecessors. The engine line-up features a 1.0-litre T-GDi (turbocharged gasoline direct injection) engine producing 100 ps, or a pair of naturally-aspirated 1.0-litre or 1.2-litre DPFI (dual-port fuel injection) engines, producing 67 ps and 84 ps, respectively.

Among other innovations, the 1.0-litre T-GDi engine features a more efficient 350 bar high-pressure fuel injection system. Meanwhile, the two naturally-aspirated engines replace multi-point injection with new dual-port fuel injection technology, with optimised injection timing for maximum fuel efficiency. They feature improved exhaust gas recirculation lines and optimised intake valve timing, for maximum fuel efficiency improvements.

All three engines are equipped with a new Integrated Thermal Management System (ITMS), which provides more effective combustion chamber cooling. ITMS varies engine cooling characteristics based on the engine’s operating point.

All three engines drive the front wheels via a five-speed manual transmission. The updated Picanto is also equipped with Kia’s new Automated Manual Transmission (AMT). The AMT is based around a five-speed manual transmission with clutch and gear shift actuators to automate clutch operation and gear shifts. Using a single dry clutch, the AMT is a cost-effective option for customers who favour the convenience and ease-of-use of an automatic transmission, without sacrificing the fuel efficiency of a conventional manual transmission. The AMT is available on both the 1.0- and 1.2-litre naturally aspirated engines.

Infotainment

UVO Connect ‘Phase II’ telematics with an upgraded touchscreen

The Picanto features a larger 8.0-inch ‘floating’ touchscreen navigation system, replacing the earlier 7.0-inch system. It also features Bluetooth multi-connection, enabling users to pair up to two mobile devices at the same time – one for hands-free phone and media use; the other for media use only. The system features Apple CarPlay and Android Auto as standard.

Kia’s new ‘Phase II’ UVO Connect telematics features include a range of on-board services including live traffic information, weather forecasts, points of interest (POIs), fuel prices, and details of potential on- and off-street parking – including price, location and availability. Online Voice Recognition lets users search for POIs, addresses, weather updates, or send text messages with voice commands. Depending on country, users can remotely send route directions to their car before a journey, check the location of their vehicle, and access vehicle reports and diagnostic notifications.

A new ‘Online Navigation’ routing service uses cloud-based real-time and historical traffic data to predict time-efficient routes more accurately than ever before.

Users can also access the accompanying UVO App, compatible with Android and Apple smartphones, which offers new ‘Phase II’ features such as Last Mile Navigation. This helps customers continue navigating to their destination after they have parked and left the car behind. It helps Kia customers navigate in towns and cities where parking hubs are often some distance away from shops, cafés and restaurants.

Ride and handling

Fun and easy to drive in town, comfortable and stable out on the open road

The Picanto combines high ride stability with quick, enjoyable handling responses, aided by independent MacPherson strut suspension at the front and a torsion beam rear axle. In addition, its relatively long wheelbase contributes to a natural improvement in ride quality and stability on all roads.

A quick steering rack enables immediate responses to driver inputs, while meaning fewer turns of the wheel from lock-to-lock than many rivals (2.8 turns), making the Picanto easy to drive at low speeds and in the city. The front wheels are pushed close to the front of the vehicle, resulting in low weight ahead of the front axle and more agile handling.

The Picanto offers torque vectoring by braking – an additional function of the Electronic Stability Control system (ESC; see ‘Safety’). The use of this new technology in an A-segment car means the Picanto retains a higher degree of handling stability and security under cornering, while helping to reduce understeer during harder cornering.

Safety

Wide range of new ADAS, safety and convenience technologies available

The upgraded Picanto features several of Kia’s latest Advanced Driver Assistance Systems (ADAS), and a range of new active safety systems to enhance occupant safety, making it one of the safest cars in its class.

Depending on specification, the Picanto’s new ADAS range includes Forward Collision-Avoidance Assist (FCA) with vehicle and pedestrian detection. Additionally, the Picanto is available with Blind-Spot Collision-Avoidance Assist (BCA), Lane Keeping Assist (LKA), Driver Attention Warning (DAW), and Lane Following Assist (LFA).

LFA uses the front-facing camera and sensors to monitor road markings, controlling the Picanto’s steering to keep the car in the centre of its lane. The Picanto also features Rear Cross-Traffic Collision-Avoidance Assist (RCCA) which alerts the driver and stops the vehicle if it detects another vehicle approaching from the left or right when reversing out of a parking space.

The Picanto is equipped with an Emergency Stop Signal (ESS), which automatically flashes the hazard warning lights to alert the car behind if the driver has to brake suddenly. Hill-start Assist Control (HAC) holds the car on the brake to ensure the car moves away smoothly from a hill start.

The updated Picanto is also equipped with Kia’s Vehicle Stability Management (VSM), Torque Vectoring By Braking, Cornering Brake Control (CBC) and Straight-line Stability (SLS) systems as standard, helping drivers maintain control under braking and cornering.

The updated Picanto is fitted with up to seven airbags throughout the cabin, including a driver’s knee airbag, depending on specification and market. It also features ISOFIX child-seat tether and anchor points.

Production and sales

On sale: updated Picanto arrives in Europe in the third quarter of 2020

The updated Kia Picanto is manufactured in Korea and has been on-sale across Europe since the third quarter of 2020. All models offer the company’s unique 7-Year, 150,000 km warranty as standard.

###

KIA PICANTO

TECHNICAL SPECIFICATIONS (EUROPE)

Body and chassis

Five-door, four- or five-seater hatchback, with all-steel unitary construction bodyshell. Choice of gasoline three- and four-cylinder engines, driving the front wheels via five-speed manual or five-speed automated manual transmission – depending on model

Engines

1.0-litre / 100 ps ‘Smartstream’ T-GDI gasoline (1.0T)

Type			Three-cylinder in-line, turbocharged, gasoline particulate filter

Capacity		1.0-litres, 998 cc

Bore and stroke	71.0 x 84.0 mm

Compression ratio 	10.0:1

Max power		100 ps (74 kW) @ 6,000 rpm

Max torque		172 Nm (127 lb ft) @ 1,500~4,000 rpm

Valves			12 (four per cylinder)

Fuel system		Direct injection

1.25-litre / 84 ps MPI gasoline (1.25)

Type			Four-cylinder in-line, naturally-aspirated

Capacity		1.25-litres, 1,248 cc

Compression ratio	11.0:1

Bore and stroke	71.0 x 78.8 mm

Max power		84 ps (62 kW) @ 6,000 rpm

Max torque		118 Nm (87 lb ft) @ 4,200 rpm

Valves			16 (four per cylinder)

Fuel system		Dual-port injection

1.0-litre / 67 ps MPI gasoline (1.0)

Type			Three-cylinder in-line, naturally-aspirated

Capacity		1.0-litres, 998 cc

Compression ratio	11.0:1

Bore and stroke	71.0 x 84.0 mm

Max power		67 ps (49 kW) @ 5,500 rpm

Max torque		96 Nm (71 lb ft) @ 3,750 rpm

Valves			12 (four per cylinder)

Fuel system		Dual-port injection

Transmissions

MT: Five-speed manual transmission

AMT: Five-speed automated manual transmission

1.0T		1.25		1.0

MT			Standard	Standard	Standard

AMT			---		Optional 	Optional

Gear ratios

Four-seat model

1.0T		1.0

MT		MT		AMT

1			3.615		3.545		3.909

2			1.955		1.895		2.056

3			1.207		1.192		1.269

4			0.893		0.906		0.964

5			0.688		0.719 / 0.744*	0.774

Reverse		3.545		3.636		3.636

Final drive		3.647		4.438 / 3.909*	4.438

*without Idle Stop&Go (ISG) / with ISG

Five-seat model

1.0T		1.25				1.0

MT		MT		AMT		MT	AMT

1			3.615		3.545		3.545		3.909	3.909

2			1.955		1.895		1.895		2.056	2.056

3			1.207		1.192		1.192		1.269	1.269

4			0.893		0.906 / 0.853*	0.906		0.964	0.964

5			0.688		0.719 / 0.697*	0.719		0.774	0.774

Reverse		3.545		3.636		3.636		3.636	3.636

Final drive		3.647		4.235		4.235		4.235	4.438

*without Idle Stop&Go (ISG) / with ISG

Drivetrains

Front-wheel drive (all models)

Steering

Type			Electric motor-driven rack-and-pinion power steering

Steering ratio		14.3:1

Turns (lock-to-lock)	2.8

Turning circle (m)	9.4

Suspension and damping

Front	Fully independent by subframe-mounted MacPherson struts, coil springs and gas-filled shock absorbers, with anti-roll stabiliser bar

Rear	Semi-independent by CTBA (coupled torsion beam axle), coil springs and gas-filled shock absorbers, with anti-roll stabiliser bar

Brakes

Type, front		256 mm solid disc

Type, rear		203 mm drum

Wheels and tyres

Standard		Steel 14-inch, 175/65 R14 tyres

Optional 		Alloy 14-inch, 175/65 R14 tyres

Alloy 15-inch, 185/55 R15 tyres

Alloy 16-inch, 195/45 R16 tyres

Spare			Tyre mobility kit or optional temporary spare wheel

Dimensions (mm)

Exterior

Overall length		3,595		Overall width*		1,595

Overall height		1,485		Wheelbase		2,400

Front overhang	675		Rear overhang	520

Front track		1,420		Rear track   		1,423

Ground clearance	141

*excluding door mirrors

Interior

1st row		2nd row

Head room		1,005		960

Leg room		1,085		820

Shoulder room	1,300		1,280

Hip room		1,245		1,230

Capacities

Fuel tank		35 litres

Luggage (VDA)	255 litres

1,010 litres (with rear seats folded down)

Weights (kg)

Four-seat model

1.0T		1.0

MT		MT	AMT

Curb weight		1,050		996	998

Gross weight		1,375		1,320	1,320

Five-seat model

1.0T		1.25			1.0

MT		MT	AMT		MT	AMT

Curb weight		1,057		1,019	1,019		1,002	1,004

Gross weight		1,450		1,415	1,415		1,400	1,405

Performance

1.0T		1.25			1.0

MT		MT	AMT		MT	AMT

Top speed (km/h)	180		173	172		161	159

0-100 km/h		10.3		12.5	15.7		14.6	17.2

80-120 km/h		9.2		14.2	8.9		14.9	9.7

Fuel efficiency and emissions (WLTP)*

1.25 (five-seat)

iMT		AMT

Fuel economy		5.2~5.6	5.4~5.7

(combined,

L/100km)

CO2 emissions	117~127	122~130

(combined, g/km)

1.0 ISG (four-seat)		1.0 ISG (five-seat)

MT		AMT		MT		AMT

Fuel economy		4.8~5.2	5.2~5.5	5.0~5.5	5.2~5.5

(combined,

L/100km)

CO2 emissions	109~118	117~125	113~124	117~125

(combined, g/km)

1.0T (four-seat)		1.0T (five-seat)

MT				MT

Fuel economy		5.0~5.5			5.0~5.5

(combined,

L/100km)

CO2 emissions	114~124			114~124

(combined, g/km)

*Fuel economy and emissions: driving range standards are calculated using the World Harmonised Light Vehicle Test Procedure (WLTP).

FEBRUARY 2021

- Ends -

About Kia Motors Europe

Kia Motors Europe is the European sales, marketing and service arm of the Kia Motors Corporation. With its headquarters in Frankfurt, Germany, it covers 39 markets across Europe and the Caucasus.

About Kia Motors Corporation

Kia Motors (www.kia.com) is a maker of world-class quality vehicles for the young-at-heart.

Founded in 1944, Kia today sells around 3 million vehicles a year in over 190 countries, with 52,000 employees worldwide, annual revenues of over KRW 58 trillion (about US$50 billion) and manufacturing facilities in six countries. Kia is spearheading the transition to electric vehicles and pursuing mobility services to help tackle urban challenges around the world. The company's brand slogan – "The Power to Surprise" – represents Kia’s global commitment to surprise the world with exciting, inspiring experiences that go beyond expectations.

For more information, please visit the Kia Motors Global Media Center at www.kianewscenter.com.